# PCB CANBUS Communication regulation

PCB CANBUS protocol

- Physics port

The baud rate is definable, the default is 500K, 11-bit standard identifier (ID) for CAN2.0b inner box. That is, when the host sends commands, the BMS will respond to the corresponding information. The host sends the corresponding identifier data frame command, and the BMS responds to the corresponding data frame data according to the identifier. The data frame sent by the host can carry any data. It is recommended to carry a length of 0X5A ;For example: the host sends CAN_ID = 0X100, data 0x5A; BMS will respond to the corresponding data.

Identifier Description

Refers to the description of the BMS response data. The verification uses CRC-16 for verification. For the specific verification method, see the verification description. All transmitted bytes are high byte first and low byte last.

| identifier | contents | Data length | illustrate |
| --- | --- | --- | --- |
| 0x100 | Total voltage,current,remaining capacity | 6+2BYTE verification | BYTE0~1 is the total voltage, unsigned type, high byte first, the unit is 10mV |
|  |  |  | BYTE2~3 is current, symbol type, unit is 10mA, charge is positive, discharge is negative |
|  |  |  | BYTE4~5 is the remaining capacity, unsigned, and the unit is 10mAh. |
|  |  |  | YTE6~7 is the CRC_16 check value, which is the CRC-16 check value of BYTE0~BYTE5 |
| 0x101 | Full capacity, cycle times,RSOC、 | 6+2BYTE verification | BYTE0~1 is full capacity, unsigned type, high byte first, unit is 10mAh |
|  |  |  | BYTE2~3 is the number of discharge cycles None, symbol type, the unit is 1 time |
|  |  |  | BYTE4~5 is the percentage of remaining capacity (RSOC), unsigned, and the unit is % |
|  |  |  | BYTE6~7 is the CRC_16 check value, which is the CRC-16 check value of BYTE0~BYTE5 |
| 0x102 | Balance status low byte, balance status high byte, protection status | 6+2BYTE verification | BYTE0~1 is the balance status flag of cell1~cell16, 1 is open, 0 is not open |
|  |  |  | BYTE2~3 is the balance status flag of cell17~cell33, 1 is open, 0 is not open |
|  |  |  | BYTE4~5 are protection marks, see Note 1 for details. |
|  |  |  | BYTE6~7 is the CRC_16 check value, which is the CRC-16 check value of BYTE0~BYTE5 |
| 0x103 | FET control status, production date, software version | 6+2BYTE verification | BYTE0~1 is the flag word of MOS tube status, see note 2 for details |
|  |  |  | BYTE2~3 is manufacture date, see note 3 |
|  |  |  | BYTE4~5 is soft version， |
|  |  |  | BYTE6~7 is the CRC_16 check value, which is the CRC-16 check value of BYTE0~BYTE5 |
| 0x104 | Series of battery, NTC numbers | 2+2BYTE verification | BYTE0 is the number of battery strings |
|  |  |  | BYTE1 is the number of NTC probes |
|  |  |  | BYTE2~3 is the CRC_16 check value, which is the CRC-16 check of BYTE1~BYTE2 |
| 0x105 | NTC1~NTC3 temperature value | 6+2BYTE verification | BYTE0~1 is NTC1 temperature value，Unsigned type, unit 0.1K See note 4 for details |
|  |  |  | BYTE2~3 is NTC2temperature value，Unsigned type, unit 0.1K See note 4 for details |
|  |  |  | BYTE4~5 is NTC3 temperature value，Unsigned type, unit 0.1K See note 4 for details |
|  |  |  | BYTE6~ is the CRC_16 check value, which is the CRC-16 check of ，BYTE0~BYTE5 |
| 0x106 | NTC4~NTC6 temperature value | 6+2BYTE verification | BYTE0~1 is NTC4 temperature value，Unsigned type, unit 0.1K See note 4 for details |
|  |  |  | BYTE2~3 is NTC5 temperature value，Unsigned type, unit 0.1K See note 4 for details |
|  |  |  | BYTE4~5 is NTC6 temperature value，Unsigned type, unit 0.1K See note 4 for details,if there is not ,no datas can be returned |
|  |  |  | BYTE6~7 is CRC_16 check value，which is the CRC-16 check of BYTE0~BYTE5 |
| 0x107 | CELL1~CELL3 voltage value | 6+2BYTE verification | BYTE0~1 is Cell1 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE2~3 is Cell2 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE4~5 is Cell3 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE6~7 is CRC_16 check value，which is the CRC-16 check of BYTE0~BYTE5 |
| 0x108 | CELL4~CELL6 voltage value | 6+2BYTE verification | BYTE0~1 is Cell4 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE2~3 is Cell5 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE4~5 is Cel6 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE6~7 is CRC_16 check value，which is the CRC-16 check of BYTE0~BYTE5 |
| 0x109 | CELL7~CELL9 voltage value | 6+2BYTE verification | BYTE0~1 is Cell7 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE2~3 is Cell8 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE4~5 is Cell9 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE6~7 is CRC_16 check value，which is the CRC-16 check of BYTE0~BYTE5 |
| 0x10A | CELL10~CELL12 voltage value | 6+2BYTE verification | BYTE0~1 is Cell10 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE2~3 is Cell11 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE4~5 is Cell12 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE6~7 is CRC_16 check value，which is the CRC-16 check of BYTE0~BYTE5 |
| 0x10B | CELL13~CELL15 voltage value | 6+2BYTE verification | BYTE0~1 is Cell13 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE2~3 is Cell14 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE4~5 is Cell15 voltage，Unsigned type，High byte first, the unit is mV |
|  |  |  | BYTE6~7 is CRC_16 check value，which is the CRC-16 check of BYTE0~BYTE5 |
| 0x10C | CELL16~CELL18 voltage value | 6+2BYTE verification | BYTE0~1 is Cell16 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE2~3 is Cell17 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE4~5 is Cell18 voltage ,Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE6~7 is for CRC_16 check value，is for CRC-16 check of BYTE0~BYTE5 |
| 0x10D | CELL19~CELL21 voltage value | 6+2BYTE voltage value | BYTE0~1 is Cell19 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE2~3 is Cell20 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE4~5 is Cell21 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE6~7 is CRC_16 check value， is for CRC-16 check of BYTE0~BYTE5 |
| 0x10E | CELL22~CELL24 voltage value | 6+2BYTE verification | BYTE0~1 is Cell22 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE2~3 is Cell23 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE4~5 is Cell24 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE6~7 is CRC_16 check value，for BYTE0~BYTE5 CRC-16 check |
| 0x10F | CELL25~CELL27 voltage value | 6+2BYTE verification | BYTE0~1 is Cell25 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE2~3 is Cell26 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE4~5 is Cell27 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE6~7 is for CRC_16 check value，for BYTE0~BYTE5 CRC-16 check |
| 0x110 | CELL28~CELL30 voltage value | 6+2BYTE verification | BYTE0~1 is Cell28 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE2~3 is Cell29 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE4~5 is Cell30 voltage，Unsigned type，High byte first，Unit is mV |
|  |  |  | BYTE6~7 is CRC_16 check value，for BYTE0~BYTE5 CRC-16 check |
| 0x111 | Hardware device type1 | 8BYTE |  |
| 0x112 | Hardware device type2 | 8BYTE |  |
| 0x113 | Hardware device type3 | 8BYTE |  |
| 0x114 | Hardware device type4 | 8BYTE |  |
| … | … |  | Reserved in the middle |
| 0x12D | Setting MOSswitch | 4BYTE | BYTE0: discharge MOSswitch to control，BYTE1:Inverse code of discharge switch control BYTE2: charge MOS switch to control，BYTE3:Inverse code of discharge switch control When it is 0, the MOS is turned on, when it is 1, the MOS is turned off, and after the instruction is turned off, the MOS must be turned on. |
| 0x12e | Enter factory mode | 1 | 5678 |
| 0x12f | Initialization EEPROM | Initialization parameters | Sending data 0x2828 the parameter to EEPROM |
| 0x130 | Protection parameter 1-4 | 8BYTE | Design capacity, cycle capacity, full charge voltage, cut-off voltage / If the length is 1, the data is 5A, it means read, if the length is 8, it means write. |
| 0x131 | Protection parameter 5-8 | 8BYTE | Self-discharge rate, date of manufacture, serial number, number of cycles |
| 0x132 | Protection parameter 9-12 | 8BYTE | Charging over temperature, charging over temperature recovery, charging low temperature, charging low temperature recovery |
| 0x133 | Protection parameter 15-16 | 8BYTE | Discharge over temperature, discharge over temperature recovery, discharge low temperature, discharge low temperature recovery |
| 0x134 | Protection parameter 17-20 | 8BYTE | Pack overvoltage, pack overvoltage recovery, pack low voltage, pack low voltage recovery |
| 0x135 | Protection parameter 21-24 | 8BYTE | Cell overvoltage, cell overvoltage recovery, cell undervoltage, cell undervoltage recovery |
| 0x136 | Protection parameter 25-28 | 8BYTE | Charging over-current, discharging over-current, balanced opening voltage, balanced voltage difference |
| 0x137 | Protection parameter 29-32 | 8BYTE | Current-sense resistor, function configuration, NTC configuration, number of battery strings |
| 0x138 | Protection parameter 33-36 | 8BYTE | Fet time, led time, 80 voltage. 60 voltage |
| 0x139 | Protection parameter 37-40 | 8BYTE | 40 voltage, 20 voltage, hardware overvoltage, hardware undervoltage |
| 0x13A | Protection parameter 41-44 | 8BYTE | Hardware overcurrent, hardware short circuit, hardware delay, short circuit release delay, charging low temperature delay Charge high temperature delay, discharge low temperature delay, discharge high temperature delay |
| 0x13B | Protection parameter 45-48 | 8BYTE | Pack undervoltage delay, pack overvoltage delay, monomer undervoltage delay, monomer overvoltage delay Charge over-current delay, charge over-current release, discharge over-current delay, discharge over-current release |
| 0x13C | Protection parameter 49-52 | 8BYTE | Manufacturer name 1 |
| 0x13D | Protection parameter 53-56 | 8BYTE | Manufacturer name 2 |
| 0x13E | Protection parameter 57-60 | 8BYTE | Manufacturer name 3 |
| 0x13F | Protection parameter 61-64 | 8BYTE | Manufacturer name 4 |
| 0x140 | Protection parameter 65-68 | 8BYTE | Device Name 1 |
| 0x141 | Protection parameter 69-72 | 8BYTE | Device name 2 |
| 0x142 | Protection parameter 73-76 | 8BYTE | Device Name 3 |
| 0x143 | Protection parameter 77-80 | 8BYTE | Device Name 4 |
| 0x144 | Protection Parameter 81-84 | 8BYTE | barcode 1 |
| 0x145 | Protection Parameter 85-88 | 8BYTE | barcode 2 |
| 0x146 | Protection Parameter 89-92 | 8BYTE | barcode 3 |
| 0x147 | Protection Parameter 93-96 | 8BYTE | barcode 4 |
| 0x148 | Protection Parameter 97-100 | 8BYTE | Charge time,、GPS voltage、gps delay、90 voltage |
| 0x149 | Protection Parameter 101-104 | 8BYTE | 70 voltage、50 voltage、30 voltage、10 voltage |
| 0x14A | Protection Parameter 105-108 | 2BYTE | 100 voltage、others |

- Special Instructions

Note 1: The protection indicates ，1 indicates that protection has occurred,  0 has not occurred

| bit0 | Single overvoltage protection |
| --- | --- |
| bit1 | Single undervoltage protection |
| bit2 | Pack overvoltage protection |
| bit3 | Pack overvoltage protection |
| bit4 | Charge over temperature protection |
| bit5 | Charge low temperature protection |
| bit6 | Discharge over temperature protection |
| bit7 | Discharge low temperature protection |
| bit8 | Charge overcurrent protection |
| bit9 | Discharge overcurrent protection |
| bit10 | Short circuit protection |
| bit11 | Front detection IC error |
| bit12 | Software lock MOS |
| bit13~bit15 | Reserved |

Note2：Note 2: MOSFET status:  bit 0 for charging MOS, bit 1 for discharge MOS, 0 for MOS off, 1 for open

Note 3：Production date analysis

Using 2 bytes of transmission;For example, 0x2068, where the date is the lowest 5: 0x2028&0x1f = 8             represent date; month 4b bit (0x2068 >> 5) & 0x0f = 0x03 represent March; The year is expressed in terms of the difference from the year 2000: 2000 + (0x2068 >> 9 ) = 2000 + 0x10 = 2016;

Note4：Temperature Analysis

Using absolute temperature transmission, send value = 2731 + (actual temperature * 10),

0℃ = 2731 +0*10

25℃ = 2731+25*10 = 2981

-10℃ = 2731 +（-10）*10= 2631

3.The verification method in this agreement adopts CRC-16 check, the specific verification method is as follows:

Calculation Method 1：

#define         CRC_16_POLYNOMIALS  0xa001    
// X^16 +X^15 +X^2+1 ¶àÏîÊ½
uint16_t Check_CRC16(uint8_t* pchMsg,uint8_t wDataLen)
{
    uint8_t i, chChar;
    uint16_t wCRC = 0xFFFF;
    while (wDataLen--)
    {
        chChar = *pchMsg++;
        wCRC ^= (uint16_t) chChar;
        for (i = 0; i < 8; i++)
        {
            if (wCRC & 0x0001)
                wCRC = (wCRC >> 1) ^ CRC_16_POLYNOMIALS;
            else
                wCRC >>= 1;
       }
   }
   return wCRC;
}

Calculation Method 2：

/* CRC CRC high byte value table */

byte[] auchCRCHi = newbyte[]{0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0,0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40

};

/* CRClow byte value table*/

byte[] auchCRCLo = newbyte[]{

0x00, 0xC0, 0xC1, 0x01, 0xC3, 0x03, 0x02, 0xC2, 0xC6, 0x06, 0x07, 0xC7, 0x05, 0xC5, 0xC4, 0x04, 0xCC, 0x0C, 0x0D, 0xCD, 0x0F, 0xCF, 0xCE, 0x0E, 0x0A, 0xCA, 0xCB, 0x0B, 0xC9, 0x09, 0x08, 0xC8, 0xD8, 0x18, 0x19, 0xD9, 0x1B, 0xDB, 0xDA, 0x1A, 0x1E, 0xDE, 0xDF, 0x1F, 0xDD, 0x1D, 0x1C, 0xDC, 0x14, 0xD4, 0xD5, 0x15, 0xD7, 0x17, 0x16, 0xD6, 0xD2, 0x12, 0x13, 0xD3, 0x11, 0xD1, 0xD0, 0x10, 0xF0, 0x30, 0x31, 0xF1, 0x33, 0xF3, 0xF2, 0x32, 0x36, 0xF6, 0xF7, 0x37, 0xF5, 0x35, 0x34, 0xF4, 0x3C, 0xFC, 0xFD, 0x3D, 0xFF, 0x3F, 0x3E, 0xFE, 0xFA, 0x3A, 0x3B, 0xFB, 0x39, 0xF9, 0xF8, 0x38, 0x28, 0xE8, 0xE9, 0x29, 0xEB, 0x2B, 0x2A, 0xEA, 0xEE, 0x2E, 0x2F, 0xEF, 0x2D, 0xED, 0xEC, 0x2C, 0xE4, 0x24, 0x25, 0xE5, 0x27, 0xE7, 0xE6, 0x26, 0x22, 0xE2, 0xE3, 0x23, 0xE1, 0x21, 0x20, 0xE0, 0xA0, 0x60, 0x61, 0xA1, 0x63, 0xA3, 0xA2, 0x62, 0x66, 0xA6, 0xA7, 0x67, 0xA5, 0x65, 0x64, 0xA4, 0x6C, 0xAC, 0xAD, 0x6D, 0xAF, 0x6F, 0x6E, 0xAE, 0xAA, 0x6A, 0x6B, 0xAB, 0x69, 0xA9, 0xA8, 0x68, 0x78, 0xB8, 0xB9, 0x79, 0xBB, 0x7B, 0x7A, 0xBA, 0xBE, 0x7E, 0x7F, 0xBF, 0x7D, 0xBD, 0xBC, 0x7C, 0xB4, 0x74, 0x75, 0xB5, 0x77, 0xB7, 0xB6, 0x76, 0x72, 0xB2, 0xB3, 0x73, 0xB1, 0x71, 0x70, 0xB0, 0x50, 0x90, 0x91, 0x51, 0x93, 0x53, 0x52, 0x92, 0x96, 0x56, 0x57, 0x97, 0x55, 0x95, 0x94, 0x54, 0x9C, 0x5C, 0x5D, 0x9D, 0x5F, 0x9F, 0x9E, 0x5E, 0x5A, 0x9A, 0x9B, 0x5B, 0x99, 0x59, 0x58, 0x98, 0x88, 0x48, 0x49, 0x89, 0x4B, 0x8B, 0x8A, 0x4A, 0x4E, 0x8E, 0x8F, 0x4F, 0x8D, 0x4D, 0x4C, 0x8C, 0x44, 0x84, 0x85, 0x45, 0x87, 0x47, 0x46, 0x86, 0x82, 0x42, 0x43, 0x83, 0x41, 0x81, 0x80, 0x40

};

//check function

UInt16 crc16(byte[] puchMsg, UInt16 usDataLen)

{

UInt16 clc;

UInt16 i = 0;

byte uchCRCHi = 0xFF; /* High CRC byte initialization */

byte uchCRCLo = 0xFF; /* Low CRC byte initialization */

byte uIndex; /* Index in CRC cycle */

while (usDataLen-- > 0) /* transmit message buffer */

{

uIndex = (byte)(uchCRCHi ^ puchMsg[i++]); /* 计Calculation CRC */

uchCRCHi = (byte)(uchCRCLo ^ auchCRCHi[uIndex]);

uchCRCLo = auchCRCLo[uIndex];

}

clc = (UInt16)(uchCRCHi << 8 | uchCRCLo);

return (clc);

}